3G/HD/SD-SDI to HDMI Converter

BAT-SH

(1) 기 능

본 기기는 3G/HD/SD-SDI 신호를 HDMI 신호로 변환하는 컨버터 장비로서, 최저의 SDI Jitter성능을 갖고 있어 뛰어난 신호품질을 제공하여 사용자가 잡음이 들어가있는 아날로그 비디오/오디오 신호를 SDI케이블을 통해서 멀리 좋은 품질로 보낼 수 있어야 하고 다음과 같은 사양에 준한다.

- 효율적인 열 발산설계

- 타 브랜드와 호환성

- 다양한 오디오 샘플링 지원(32~192kHz to 48kHz)

- 다양한 입력전원 지원(+7V~+15V)

- 저전력 소모(3Watt Max.)

- SDI　신호 자동 검색 - 3G/HD/SD

- 별도의 Analog Stereo Audio (RCA) 출력지원

- HDMI ｖ1.4 지원

- SDI　Re-clocked Loop Through 지원

(2) 기기 특성

- Supported Video Formats: 720(1440) x 480i @ 59.94/60, 720(1440) x 576i @ 50

1280 x 720p @ 50, 1280 x 720p @ 59.94/60

1920 x 1080i @ 50, 1920 x 1080i @ 59.94/60

1920 x 1080p @ 23.94/24, 1920 x 1080p @ 25,

1920 x 1080p @ 29.97/30

1920 x 1080p @ 50, 1920 x 1080p @ 59.94/60

- Video Input: 3G, HD, and SD-SDI(auto-selected),

SMPTE-259/272/274/292/296/299/424/425

1 x BNC input, 1x BNC Loop-through Output

- Video Output: HDMI v1.3a, 24/30/36 bits per pixel, RGB, 2.25Gbs, SD, HD, HDMI Standard Type A

connector

- Audio Outputs: HDMI embedded audio, 24 bit, 2 or 8

channels RCA-style analog outputs at -10dBV

- Input Power Range: +7V ~ +15V

- Power Consumption: 3Watt [Max]

- Power Adapter: Input : 100V ~ 240V, 50/60Hz,

Output : +12V DC, 2.0A

- BAT-SH 및 동등 이상 품이어야 한다.